This document contains information regarding the organization, formats, and structure of the PyPSA_BC GitHub repository.

All assets (generators, transmission lines, and storage assets) all have unique identifiers called an “asset_id”. Most asset_id’s have been derived from CODER’s component ID and connecting node codes.

Buses

Busses are used to represent a flow between generators, stores, links, and lines of a specific carrier at a specific node (i.e. 230 kV electricity at a substation connected to the transmission grid and a generator).

All buses take on the following general naming format:

“{IDENTIFIER} {CARRIER} Bus“

- Identifier: Takes on a string which helps to distinguish the Bus from other buses with the same carrier type.

- Carrier: The energy vector (i.e. electricity, energy for heating, water, etc..)

- Bus: Static string indicating the component is a bus.

| Type | Name (string) | Remarks |
| --- | --- | --- |
| Water Bus | “{ASSET_ID} Water Bus“ |  |
| Electrical Bus | “{CONNECTING_NODE_CODE} ELC-{TYPE} Bus” |  |
| Heat Bus | “{CONNECTING_NODE_CODE} HEAT-{TYPE} Bus” |  |
| Gas Bus | “{ASSET_ID} {FUEL TYPE} Bus” |  |

Generators